COMPUTER ORGANISATION & ARCHITECTURE LAB – IV

Datapath and Control Unit Design — Single-Cycle RISC-V Processor

In this Lab you will build a working single-cycle RISC-V CPU simulator in C/C++, one layer at a time. The design must be MODULAR: the datapath (Session 1) and the control unit (Session 2) are separate pieces of code with a clean interface between them. You must be able to swap or re-test either half without rewriting the other.

Supported Instruction Subset

Your simulator only needs to support the following seven RV32I instructions. Do not implement shifts, multiply/divide, floating point, or any instruction outside this list.

| Instruction | Type | Meaning |
| --- | --- | --- |
| add rd, rs1, rs2 | R-type | rd = rs1 + rs2 |
| sub rd, rs1, rs2 | R-type | rd = rs1 - rs2 |
| and rd, rs1, rs2 | R-type | rd = rs1 & rs2 |
| or rd, rs1, rs2 | R-type | rd = rs1 ¦ rs2 |
| lw rd, offset(rs1) | I-type | rd = Mem[rs1 + offset] |
| sw rs2, offset(rs1) | S-type | Mem[rs1 + offset] = rs2 |
| beq rs1, rs2, offset | B-type | if (rs1 == rs2) PC = PC + offset |

Note: This deliberately uses lw/sw (RV32 word load/store) rather than the ld/sd (RV64 doubleword) shown in the reference slides — your earlier assembler lab targets 32-bit RISC-V, so the simulator should match it. Confirm this with your instructor if in doubt.

Instruction Encoding Reference

Refer to the RISC-V Green Card and the CPU Design slide deck for full field layouts. Quick reference for this subset:

| Instruction | opcode | funct3 | funct7 |
| --- | --- | --- | --- |
| add | 0110011 | 000 | 0000000 |
| sub | 0110011 | 000 | 0100000 |
| and | 0110011 | 111 | 0000000 |
| or | 0110011 | 110 | 0000000 |
| lw | 0000011 | 010 | — |
| sw | 0100011 | 010 | — |
| beq | 1100011 | 000 | — |

Assignment 1 — Datapath Components (Session 1)

In this session you will implement each datapath component as an independent, testable module. Do NOT wire them together with real decision-making logic yet — you are building and unit-testing the building blocks.

1.1 — Modularity requirement

Each component above must be its own function/struct/class with a clearly defined interface (inputs in, outputs out). Keep them in separate files or clearly separated sections (e.g. datapath.h / datapath.c). The control unit you build in Assignment 2 must be able to plug into these exact interfaces without you having to modify their internals.

1.2 — Components to implement

- Program Counter (PC) + Adder: holds current instruction address; a separate adder computes PC + 4.

- Instruction Memory: an array holding machine code; given an address, returns the 32-bit instruction.

- Register File: 32 general-purpose registers (x0 hardwired to 0). Must support two simultaneous reads (rs1, rs2) and one write (rd), gated by a RegWrite signal.

- Immediate Generator: given the raw instruction bits, extracts and sign-extends the immediate for I-type, S-type, and B-type formats.

- ALU: takes two operands and an operation selector; supports at minimum ADD, SUB, AND, OR; produces a result and a Zero flag.

- Data Memory: byte-addressable memory array; supports word read (MemRead) and word write (MemWrite), gated by separate signals — MemRead and MemWrite must never both be asserted for the same access.

1.3 — Tasks

- Implement all six components listed in 1.2.

- Write a small standalone test driver (not the full CPU) that manually sets control signals and operand values, and calls each component directly to prove it behaves correctly — e.g. write a value into the register file and read it back; run the ALU on a few operand pairs and check results; write and read back a word from data memory.

- For at least one instruction of each type in the supported subset (R, I, S, B), show by hand which bits of the instruction correspond to opcode, funct3, funct7, rs1, rs2, rd, and immediate. Attach this as a short table or annotated diagram.

- Draw (by hand or any tool) a block diagram of your datapath components and how they will eventually connect — labelling every wire you expect to carry a control signal (RegWrite, MemRead, MemWrite, ALU operation select, etc.), even though those signals are not yet driven by a control unit.

Assignment 2 — Control Unit (Session 2)

In this session you implement the control unit as a separate module and connect it to the datapath components you already built. The datapath code from Assignment 1 should not need to change — only the control unit is new, and it drives the signals that were left unconnected in Session 1.

2.1 — What the control unit does

Given an instruction's opcode (and funct3/funct7 where needed for R-type), the control unit must produce the following signals:

| Signal | Purpose |
| --- | --- |
| RegWrite | 1 if the instruction writes a result back to the register file |
| ALUSrc | selects ALU's second operand: register value (R-type) or immediate (I/S-type) |
| ALUOp / ALU control | selects which ALU operation to perform (add, sub, and, or) — for R-type this depends on funct3+funct7; for lw/sw it is always add; for beq it is sub |
| MemRead | 1 only for lw |
| MemWrite | 1 only for sw |
| MemtoReg | selects register write-back source: ALU result (R-type) or memory read data (lw) |
| Branch | 1 only for beq; combined with the ALU's Zero flag decides whether to branch |

2.2 — Tasks

- Implement the control unit as its own module (e.g. control.h / control.c), taking opcode (+funct3/funct7) as input and producing the signal set in 2.1 as output.

- Wire the control unit's outputs into the Assignment 1 datapath components' existing signal inputs. Do not modify how the datapath components work internally — only connect signals that were previously hardcoded/manual in your Assignment 1 test driver.

- Write a driver that takes ONE encoded instruction (as a 32-bit hex value) at a time, runs it through decode → control unit → datapath, and prints the resulting register/memory change. Test this for at least one instruction of each of the 7 supported types.

- For each of the 7 instructions, produce a filled-in control signal table (one row per instruction, one column per signal from 2.1) showing what your control unit outputs.

2.3 — Submit

- Control unit source code.

- Updated integration code showing datapath + control unit wired together.

- The completed control signal table for all 7 instructions.

- Test output for at least one instruction of each type, run individually.

Assignment 3 — Full Program Execution & Verification (Session 3)

In this session you combine everything into a working fetch–decode–execute loop that runs a complete program — the same machine code your assembler (from the earlier lab) produces — and verify your simulator's output against a reference tool.

3.1 — Machine code input format

Your instruction memory should be loaded from a plain text file: one 32-bit instruction per line, written in hex (e.g. 00A28293), in the order they should be executed. This is the format your assembler should emit. If your assembler currently emits something else (raw binary, a different text layout), convert it or adjust your reader accordingly — check with your assembler-lab groupmates if unsure.

3.2 — Tasks

- Implement the fetch–decode–execute loop: fetch the instruction at PC, decode it, run it through your control unit and datapath, update PC (sequential PC+4, or branch target if beq is taken), and repeat until the end of the instruction memory is reached.

- Write (or reuse from the assembler lab) a short RV32I test program using only the 7 supported instructions — it should include at least one lw, one sw, one beq (taken), one beq (not taken), and at least two of add/sub/and/or.

- Run your test program through your simulator. Print the final state of all 32 registers and any data memory locations touched by the program.

- Verification: Load the same program into the RISC-V Visualizer (https://risc-v-cpu-visualizer.vercel.app/single-stage) and step through it there. This tool is a reference single-cycle simulator — use it only to check your own simulator's correctness, not as your submitted implementation. Compare its final register/memory state to your simulator's output after every instruction, or at minimum at the end of the program. Attach screenshots of the visualizer's state alongside your own simulator's output.

- If your output differs from the visualizer's at any point, identify and fix the bug. Briefly describe (2–4 lines) what was wrong and in which module (datapath or control) the bug was.

3.3 — Submit

- Full simulator source code (datapath + control + fetch-decode-execute driver).

- The test program (assembly and/or its assembled hex).

- Final register/memory dump from your simulator.

- Screenshot(s) of the same program's final state in the RISC-V Visualizer.

- Short write-up: any mismatches found and how they were fixed (or confirmation of a first-try match).

General Notes

- Compile cleanly with warnings enabled (e.g. gcc/g++ -Wall -Wextra) — as in previous labs.

- Keep datapath and control code in separate files/modules throughout — this is checked, not optional.

- Pipelining is a separate, later assignment and is not part of this one — do not implement pipeline registers or hazard handling here.

- Comment your code, especially at module boundaries (function signatures, expected inputs/outputs).